Oasis3 System Specifications

· Core processor is a Persistor Instruments CF-2 that has 512Kb RAM, 1 Mb Flash and accepts a CF card for mass storage. Dedicated UART channel for ADCP, Radio, and Console

· 32 channels total Serial multiplexed TPU’s 

· 32 channels total Power Switching, 2 Amps each 

· 8 each 12-Bit A/D channels

Can Temp

Can Pressure

Battery Volts 

Argos Battery Volts (switched)

4 ports for science

· 8 each FET Wake-up outs, 8 source 12 vdc

· 4 each Digital I/O’s  

· Total current draw  750uA stand by, 25 mA running @12 VDC   (25% duty @ 12vdc max)

· Provide Internal mounting for existing hardware

Inductive Modem

Argos Battery Pack

· Backplane support for Opto-Isolated 232 module and GF detect PCB

· Tower mount the existing TNC/Radio so the RF subsystem may be changed in the future without internal hardware modification

· New aluminum housing and internal chassis.

· New cable and connector system to accommodate the expanded I/O

